# Lab 09

# Memory, Cache, Loops and Array

Memory Instructions: In RISC-V assembly, memory instructions like lw (load word) and sw (store word) are essential for moving data between the processor’s registers and main memory. The lw instruction loads a 32-bit value from a specified memory address into a register, allowing the CPU to perform computations on data stored in memory. Conversely, sw stores a value from a register back into memory, preserving the result for later use or for other parts of the program.

Why:

  CPU registers are limited in number and size, so most data must reside in memory.

  lw and sw allow programs to access larger datasets beyond registers.

  They enable updating variables and maintaining results across program execution.

  Facilitate efficient computation by moving data between memory and registers.

  Demonstrate a typical load-compute-store cycle:

- Load values from memory (lw)

- Compute using registers (add)

- Store results back to memory (sw)

# memory[0] = ?, memory[4] = ? lw t0, 0(x0) lw t1, 4(x0) add t2, t0, t1 sw t2, 8(x0) # store result at memory[8]

Explanation:

- RISC-V memory access uses base + offset addressing: lw t0, OFFSET(BASE_REGISTER).

- 0(x0) → base = x0 (always 0), offset = 0 → memory address = 0 → loads 1st word.

- 4(x0) → base = x0 (0), offset = 4 → memory address = 4 → loads 2nd word.

- Each .word in memory occupies 4 bytes, so addresses increment by 4 for each word: 0, 4, 8, 12…

- Example mapping:

  - memory[0] = 5 → first word

  - memory[4] = 7 → second word

  - memory[8] = result → third word

- x0 acts as a zero base, offsets point to word positions in memory.

Output

Output in memory

Explanation:

- lw t0, 0(x0): Loads the 32-bit value located at memory address 0 into register t0.

- lw t1, 4(x0): Loads the 32-bit value located at memory address 4 into register t1.

- add t2, t0, t1: Adds the two register values (from t0 and t1) and writes the sum into t2.

- sw t2, 8(x0): Stores the computed value in t2 into memory address 8.

Memory instructions with actual data.

.data num1: .word 10 # memory[0] = 10 num2: .word 20 # memory[4] = 20 result: .word 0 # memory[8] = 0 (to store result) .text la t0, num1 # load address of num1 lw t1, 0(t0) # load value 10 into t1 la t0, num2 # load address of num2 lw t2, 0(t0) # load value 20 into t2 add t3, t1, t2 # t3 = 10 + 20 la t0, result # load address of result sw t3, 0(t0) # store sum 30

Explanation:

- .data section → declares and initializes memory variables (num1 = 10, num2 = 20, result = 0).

- .text section → contains the instructions to execute.

- la t0, num1 → loads the address of num1 into register t0.

- lw t1, 0(t0) → loads the value at num1 (10) into t1.

- la t0, num2 → loads the address of num2 into t0.

- lw t2, 0(t0) → loads the value at num2 (20) into t2.

- add t3, t1, t2 → adds the two values (10 + 20) and stores the result in t3.

- la t0, result → loads the address of result into t0.

- sw t3, 0(t0) → stores the sum (30) into memory at result.

- Overall → demonstrates a load-compute-store cycle using initialized memory values.

Output in memory

Cache View (Instruction)

Cache table explanation (right side of the image)

- Index → cache line index (0–31)

- V → valid bit (1 = data present)

- D → dirty bit (1 = modified, not written back yet)

- Tag → upper address bits identifying memory block

- Word 0–3 → data in cache line (each line has 4 words)

- Line 0: contains previous data (0x10000297, 0x00028293, …)

- Line 1: contains another block of memory

- Line 2: shows your recently stored value (0x00000000 in Word 2 → likely corresponds to result)

Address mapping

- Access address: 0000...0100 = decimal 4 → corresponds to memory[4] (num2)

- Cache index calculation: lower bits of address select line → you see index 2 highlighted

- Tag: upper bits of address → helps identify which memory block is in the cache line

Statistics & plot (bottom-left)

- Hit rate: 0.7273 → ~73% of accesses found in cache

- Hits = 8, Misses = 3 → initial loads (num1, num2, result) cause misses; subsequent accesses hit cache

- Moving average shows how the hit rate evolves over cycles

Summary

- This program accesses memory 3 times for num1, num2, and result.

- Each lw/sw triggers a cache lookup, which may cause a miss or hit.

- Cache table shows how data is stored in L1, which line, which word, and if it’s dirty/valid.

- The graph tracks overall cache performance as the program runs.

Cache View (Data)

Arrays

.data myArray: .word 5, 10, 15 # Define an array with 3 numbers .text la t3, myArray # Load base address of array into t3 lw t0, 0(t3) # Load first element (5) into t0 lw t1, 4(t3) # Load second element (10) into t1 lw t2, 8(t3) # Load third element (15) into t2

Explanation:

- .data section declares myArray with 3 elements.

- la t3, myArray loads the base address of the array.

- Each lw instruction loads one element into a temporary register.

- Offsets increase by 4 bytes per word: 0, 4, 8.

Output

Data in memory

Loop example

addi t0, x0, 5 # t0 = 5 (loop counter) LOOP: addi t0, t0, -1 # t0 = t0 - 1 bne t0, x0, LOOP # If t0 != 0, repeat loop

Loop using two registers (Like in EMU8086)

addi t0, x0, 0 # counter = 0 addi t1, x0, 5 # limit = 5 LOOP_TWO: addi t0, t0, 1 # counter++ bne t0, t1, LOOP_TWO # stop at 5

Loop with condition

addi t0, x0, 5 # counter = 5 addi t1, x0, 0 # sum = 0 LOOP_COND: addi t2, x0, 3 # value to skip = 3 beq t0, t2, SKIP # if counter == 3, skip addi t1, t1, 1 # sum++ SKIP: addi t0, t0, -1 # counter-- bne t0, x0, LOOP_COND

Exercise:

Task 1: Define array, iterate using loop and computer sum of array.

Task 2: Explore stack operations in Ripes with an example code.

Task 3: Pop and push 5 values, and analyze the cache (both instruction and data).